# Received Key Processor Application - User Guide

## Table of Contents

[Introduction](#introduction)

[System Requirements](#system-requirements)

[Installation and Setup](#installation-and-setup)

[Main Features Overview](#main-features-overview)

[FDU (Field Data Unit) Viewer](#fdu-field-data-unit-viewer)

[MIDS JTRS Tag Editing](#mids-jtrs-tag-editing)

[ECU KEK ID Calculation](#ecu-kek-id-calculation)

[Binary Tag Detection](#binary-tag-detection)

[Database Management](#database-management)

[Troubleshooting Common Issues](#troubleshooting-common-issues)

## Introduction

The Received Key Processor Application (RKPA) is a desktop application designed for military and government users who need to process, manage, and analyze cryptographic key data. This application provides a comprehensive suite of tools for:

**Processing Key Files**: Convert and import 8727 binary files (.8727, .8287, .bin, .key formats)

**Viewing FDUs**: Examine Field Data Units containing key information

**Editing MIDS JTRS Tags**: Modify and validate MIDS (Joint Tactical Radio System) tags

**Managing Databases**: Organize and store cryptographic key information

**Detecting Tag Formats**: Automatically identify different tag types (JMPS, TR, MIDS JTRS, IFF, DS-100-1)

This application is built for Windows and uses a secure, layered architecture to protect sensitive cryptographic data.

## System Requirements

### Hardware Requirements

**Processor**: Intel Core i5 or equivalent (recommended)

**RAM**: 8 GB minimum, 16 GB recommended

**Disk Space**: 500 MB for application, additional space for database storage

**Display**: 1920x1080 resolution or higher

### Software Requirements

**Operating System**: Windows 10 or Windows 11

**Runtime**: .NET 8.0 Windows Desktop Runtime

**External Tool**: EKMS2.exe (must be in application directory)

### Permissions

Write access to local application data directory (for database storage)

Write access to system temporary directory (for secure file processing)

Read/write access to serial ports (if using COM port communication)

## Installation and Setup

### Step 1: Obtain the Application

Download the application package from your authorized source

Extract all files to a folder on your computer (e.g., `C:\Program Files\RKPA\`)

### Step 2: Verify External Dependencies

Ensure the following files are present in the application directory:

`EKMS2.exe` - Required for processing 8727 binary files

`ReceivedKeyProcessorApp.exe` - Main application executable

Supporting DLL files

### Step 3: Launch the Application

Navigate to the application folder

Double-click `ReceivedKeyProcessorApp.exe`

The application will launch and create the database automatically on first run

Note: The database is stored in %LOCALAPPDATA%\ReceivedKeyProcessorApp\keys.db

### Step 4: Initial Configuration

The application will automatically configure itself on first launch. Default settings include:

Database location: User's local AppData folder

Log files: Application directory

Temporary files: System temp directory with secure cleanup

## Main Features Overview

The application provides several key features accessible through the main interface:

### 1. Serial Port Communication

Receive cryptographic key data directly from radio equipment via COM port connection. This feature allows for real-time key transfer from field devices.

### 2. XML Processing

Parse and validate Tier3 XML payloads containing key information. The application validates XML structure and imports data into the database.

### 3. Database Storage

All processed key data is stored securely in a SQLite database with the following hierarchy:

**Keys** - Top-level key records

**Short Titles** - Key identifiers with material type information

**Editions** - Key versions with classification and use information

**Segments** - Actual cryptographic key data

### 4. Mission Planning

Create and manage mission plans that assign specific keys to missions. Includes validation to ensure keys are properly configured.

### 5. Duplicate Prevention

The application uses hash-based detection to prevent duplicate key data from being imported, ensuring data integrity.

### 6. Data Export

Export processed data in various formats for use with other systems or for archival purposes.

## FDU (Field Data Unit) Viewer

The FDU Viewer allows you to examine Field Data Units containing cryptographic key information.

### Opening the FDU Viewer

From the main menu, navigate to **FDU Viewer**

The FDU Viewer screen will appear

### Viewing FDU Data

The FDU Viewer displays:

**Raw Data**: Hexadecimal representation of the FDU bytes

**Decoded Information**: Human-readable interpretation of the data

**Tag Detection**: Automatic detection of tag format with confidence score

### Understanding Tag Formats

The application can detect several tag formats:

| Format | Description | Size | Maximum Score |  |
| --- | --- | --- | --- | --- |
| JMPS | Jump Tag | 16 bytes | 75% |  |
| TR | Tactical Radio with DS-100-1 | 110 bytes | 95% |  |
| MIDS JTRS | MIDS format | 103 bytes | 90% |  |
| IFF | IFF format | Variable | 80% |  |
| DS-100-1 | Transfer Key Encryption Key | 32 bytes | 100% |  |
| Composite | JMPS + DS-100-1 combined | Variable | 100% |  |

### Viewing Detection Results

When an FDU is loaded, the application displays:

**Detected Format**: The most likely tag format

**Confidence Score**: Percentage indicating how well the data matches the format

**Score Breakdown**: Detailed explanation of why the format was detected

**Warnings**: Any validation issues found

## MIDS JTRS Tag Editing

The MIDS JTRS (Joint Tactical Radio System) Tag Editor allows you to decode, view, edit, and encode MIDS JTRS tags.

### What is a MIDS JTRS Tag?

A MIDS JTRS tag is a 103-byte data structure containing:

**Control Tag** (38 bytes) - Key metadata and configuration

**Extension Tag** (65 bytes) - Additional key information and validation data

The tag includes information such as:

Internal JTRS Key ID

Key function and classification

Waveform information

Start time and crypto period

Update rules and methods

CRC validation codes

### Opening the MIDS JTRS Tag Editor

Navigate to **FDU Viewer**

Load an FDU containing MIDS JTRS tag data

The MIDS JTRS tag will be automatically detected and displayed

### Viewing Tag Information

The tag is displayed in two sections:

#### Control Tag Fields (15 fields)

| Field | Description | Range |  |
| --- | --- | --- | --- |
| Internal JTRS Key ID | Reference ID assigned by CSS | 0x00000000 - 0xFFFFFFFF |  |
| Key Function | Identifies Key Type/Usage | 0 - 63 |  |
| Classification | Classification of the Key | See Classification Values below |  |
| Compartment | Compartment restrictions | 0 - 255 |  |
| CEA Affiliation | CEA or Algorithm Identifier | See CEA Values below |  |
| Waveform Affiliation | Specific waveform instance | See Waveform Values below |  |
| Waveform Instance ID | 16 ASCII characters | Up to 16 chars |  |
| Key Slot Reference ID | 4 ASCII characters | Up to 4 chars |  |
| Start Time | Seconds since 00:00:00, 1 Jan 2000 | 0x00000000 - 0xFFFFFFFF |  |
| Crypto Period | Duration for key use | 0 - 15 |  |
| Control Method | Key expiration enforcement method | 0 - 15 |  |
| Update Method | Cryptographic update initiation rule | 0 - 15 |  |
| Update Rule | Schedule or condition for updates | 0 - 15 |  |
| Update Count | Number of times key has been updated | 0 - 255 |  |
| Control Tag CRC | CRC-16 validation (read-only) | Auto-calculated |  |

#### Extension Tag Fields (8 fields)

| Field | Description | Range |  |
| --- | --- | --- | --- |
| Internal JTRS Key ID | Replicated from Control Tag | Must match Control Tag |  |
| Source Tag Data Format Type | Format of External Key ID | 0-3 |  |
| ECU KEK ID | ECU KEK identifier | 0x00000000 - 0xFFFFFFFF |  |
| Short Title | External Key ID Short Title | Up to 24 chars |  |
| Edition | Edition name | Up to 6 chars |  |
| Register | Register number | 0 - 68719476735 |  |
| Segment Number | Segment number | 0 - 255 |  |
| Overall Tag CRC | CRC-32 validation (read-only) | Auto-calculated |  |

### Editing a Tag

#### Step 1: Enter Edit Mode

Click the **Edit** button in the top-right corner of the MIDS JTRS Tag section

The button text changes to **Save**

All editable fields become active

#### Step 2: Modify Fields

**Text Fields**: Click on the field and type the new value

**Dropdown Fields**: Select from the available options

Classification: Choose from predefined security classifications

Compartment: Select compartment restrictions

Key Function: Choose the key type

Crypto Period: Select duration (0-15)

Control Method: Choose expiration enforcement method

Update Method: Select update initiation rule

Update Rule: Choose update schedule

Waveform Affiliation: Select waveform type

Source Tag Data Format Type: Choose format type (0-3)

#### Step 3: Save Changes

Click the **Save** button

The application automatically:

Validates all field values

Recalculates CRC-16 for Control Tag

Recalculates CRC-32 for Overall Tag

Updates the tag data

A success message appears

#### Step 4: Cancel Changes (Optional)

If you need to discard your changes:

Click the **Cancel** button

All fields revert to their original values

Edit mode exits

### Important Notes

**CRC Fields**: Control Tag CRC and Overall Tag CRC are always read-only and are automatically calculated when you save

**Validation**: The application validates all fields when saving. Invalid values will be highlighted with error messages

**String Fields**: Text fields are automatically trimmed of trailing spaces and null characters

**Spare Fields**: Spare fields must remain zero. They are not editable

### Classification Values

The Classification field supports these values:

| Value | Display Name |  |
| --- | --- | --- |
| 0x00 | Not Defined |  |
| 0x03 | UNCLASSIFIED |  |
| 0x05 | FOUO (UNCLASS) |  |
| 0x06 | RESTRICTED |  |
| 0x09 | CONFIDENTIAL |  |
| 0x0A | SECRET |  |
| 0x0C | TOP SECRET |  |
| 0x0F - 0x1E | Reserved for Standardized Use |  |
| 0x21 - 0x3C | Reserved for JTRS Internal Use |  |
| 0x3F | Not Applicable |  |

### Compartment Values

The Compartment field supports these values:

| Value | Display Name |  |
| --- | --- | --- |
| 0x00 | Not Defined |  |
| 0x03 | US ONLY |  |
| 0x05 | ALLIED |  |
| 0x06 | NATO |  |
| 0x09 | NATO ATOMAL |  |
| 0x0A | INTEL SOURCES |  |
| 0x0C | COALITION |  |
| 0x0F | US AGENCY |  |
| 0x11 | PUBLIC SAFETY |  |
| 0x12 | CCEB |  |
| 0x14 | Special Access 1 |  |
| 0x17 | Special Access 2 |  |
| 0x18 | Special Access 3 |  |
| 0x1B | Special Access 4 |  |
| 0x1D | Special Access 5 |  |
| 0x1E | Coalition 2 |  |
| 0x21 | Coalition 3 |  |
| 0x22 | Coalition 4 |  |
| 0x24 | Coalition 5 |  |
| 0x27 | Coalition 6 |  |
| 0x28 | Coalition 7 |  |
| 0x2B | Coalition 8 |  |
| 0x2D - 0x7E | Reserved for Standardized Use |  |
| 0x81 - 0xFC | Reserved for JTRS Internal Use |  |
| 0xFF | Not Applicable |  |

### Waveform Affiliation Values

The Waveform Affiliation field supports these values:

| Value | Display Name |  |
| --- | --- | --- |
| 0x00 | Not Defined |  |
| 0x01 | WNW |  |
| 0x02 | EPLRS/SADL |  |
| 0x03 | SINCGARS/ESIP |  |
| 0x04 | VHF/UHF LOS (AM/FM/PSK) |  |
| 0x05 | VHF/UHF FM/LMR |  |
| 0x06 | Cellular & PCS |  |
| 0x07 | HAVE QUICK I/II |  |
| 0x08 | SATURN |  |
| 0x09 | UHF SATCOM |  |
| 0x0A | MUOS-CAI (Obj. UHF SATCOM) |  |
| 0x0B | Mobile Satellite Service (MSS) |  |
| 0x0C | Link-4A/TADIL-C |  |
| 0x0D | Link-11A/TADIL-A (UHF/HF) |  |
| 0x0E | Link-11B/TADIL-B |  |
| 0x0F | Link-16/TADIL-J |  |
| 0x10 | Link-22/NILE |  |
| 0x11 | HF-ISB ALE |  |
| 0x12 | HF-SSB ALE AJ |  |
| 0x13 | STANAG 4193 (IFF) |  |
| 0x14 | STANAG 4529 (HF NB) |  |
| 0x15 | STANAG 5066 (HF MSG) |  |
| 0x16 | DWTS |  |
| 0x17 | Soldier Radio |  |
| 0x18 | COBRA |  |
| 0x19 | IBS-M |  |
| 0x1A | BOWMAN HF |  |
| 0x1B | BOWMAN VHF |  |
| 0x1C | BOWMAN UHF |  |
| 0xFF | Non-Waveform System Key |  |

## ECU KEK ID Calculation

The ECU KEK ID (Electronic Control Unit Key Encryption Key Identifier) uniquely identifies the ECU KEK used to encrypt a key. This feature allows you to calculate and validate ECU KEK IDs.

### What is ECU KEK ID?

The ECU KEK ID is a 32-bit value calculated from:

**ShortTitle** (24 bytes) - The external key identifier

**Edition** (6 bytes) - The edition name

**Segment Number** (1 byte) - The segment number

The calculation uses a SHA-1 hash, then extracts specific bytes from the hash.

### Calculating ECU KEK ID

#### Step 1: Open MIDS JTRS Tag Editor

Navigate to **FDU Viewer**

Load an FDU containing a MIDS JTRS tag

#### Step 2: Click Calculate ECU KEK ID

In the Extension Tag section, locate the **ECU KEK ID** field

Click the **Calculate ECU KEK ID** button next to the field

#### Step 3: View the Result

A dialog will appear showing:

**ShortTitle**: Current value from the tag

**Edition**: Current value from the tag

**Segment Number**: Current value from the tag

**Calculated ECU KEK ID**: The calculated value in hexadecimal format

#### Step 4: Close the Dialog

Click the Close button to close the dialog. The calculated value is for viewing only and does not update the tag data.

### Special Values

**0x00000000**: This is a special value indicating a "Red Key" - a key that is not encrypted with an ECU KEK

### When to Use This Feature

Use this feature when you need to:

Verify that an existing ECU KEK ID is correct

Understand how an ECU KEK ID was generated

Validate that the ECU KEK ID matches the expected value for a given ShortTitle, Edition, and Segment

## Binary Tag Detection

The application automatically detects and identifies different tag formats in FDU data. This helps you understand what type of key data you are working with.

### How Detection Works

When you load an FDU, the application:

**Analyzes the Data**: Examines the byte structure and patterns

**Tests Each Format**: Tries to match the data against known tag formats

**Calculates Scores**: Assigns a confidence score to each format

**Selects Best Match**: Displays the format with the highest score

### Confidence Scoring

Each tag format has a maximum possible score:

| Format | Maximum Score | Key Validation Criteria |  |
| --- | --- | --- | --- |
| JMPS | 75% | Equipment ID, dates, ASCII text |  |
| TR | 95% | CRC validation, fixed values, ASCII text |  |
| MIDS JTRS | 90% | CRC validation, field validation |  |
| IFF | 80% | CRC validation, fixed values |  |
| DS-100-1 | 100% | CRC-8, format validation, text validation |  |
| Composite | 100% | Combines JMPS and DS-100-1 validation |  |

### Understanding Score Breakdown

The application provides a detailed breakdown of how the score was calculated. For example, for MIDS JTRS tags:

**CRC Validation (Control Tag)**: 12.5% - Checks if CRC-16 is valid

**CRC Validation (Overall Tag)**: 12.5% - Checks if CRC-32 is valid

**Source Tag Data Format**: 10% - Checks if format is correct

**Valid WaveformInstanceID**: 10% - Checks if text is valid ASCII

**Valid ShortTitle**: 10% - Checks if text is valid ASCII

**Valid Edition**: 5% - Checks if text is valid ASCII

**Spare Fields Zero**: 10% - Checks if spare fields are zero

**Internal ID Repeat Match**: 10% - Checks if IDs match

**Valid Effectivity Time**: 10% - Checks if time is reasonable

**Valid Length**: 5% - Checks if size is correct

### Viewing Detection Results

Load an FDU in the FDU Viewer

Look at the **Tag Detection** section

You will see:

**Detected Format**: The format that matched best

**Confidence Score**: The percentage score

**Score Breakdown**: Detailed explanation of the score

**Warnings**: Any validation issues

### What to Do with Low Scores

If the detected format has a low confidence score:

**Review the Warnings**: Check what validation issues were found

**Verify the Data**: Ensure you loaded the correct file

**Check File Format**: The file may not contain the expected tag format

**Consult Documentation**: Refer to format specifications for more details

## Database Management

The application stores all processed key data in a SQLite database. You can view and manage this data through the Database Viewer.

### Opening the Database Viewer

From the main menu, navigate to **Database**

The Database Viewer screen will appear

### Database Structure

The database uses a hierarchical structure:

Keys (Top Level)
  └── Short Titles
      └── Editions
          └── Segments (Contains the actual key data)

### Viewing Data

The Database Viewer provides three views:

#### Hierarchical View

Displays the data in a tree structure:

Expand a **Key** to see its Short Titles

Expand a **Short Title** to see its Editions

Expand an **Edition** to see its Segments

Click on any item to see its details

#### Table View

Displays data in a tabular format with columns for each field. Use this view to:

See multiple records at once

Sort by any column

Filter data

#### Details View

Shows complete information for a selected record including:

All field values

Classification information

Key use information

Segment data

### Searching and Filtering

#### Search

Use the **Search** box at the top of the screen

Type your search term

Results are filtered as you type

#### Filter

Click the **Filter** button

Select filter criteria:

Classification

Key Use

Date Range

Material Type

Click **Apply** to filter

### Adding Test Data

The application can generate test data for testing purposes:

In the Database Viewer, click **Add Test Data**

The application will generate sample records

Wait for the operation to complete

Note: This feature is for testing only and should not be used in production environments.

### Send to SKL

You can send cryptographic key segments directly to an SKL (Single Key Loader) device from the database hierarchical view.

Access: Database page → Expand hierarchical view → Right-click on segment(s) → "Send to SKL"

What happens:

An output window appears showing real-time progress

The application generates 8727 binary content with Issue FDU (0x02B0)

A temporary file is created (`.8727.bin` extension)

The content is sent to SKL via load.exe using DS-101 protocol with discrete FDU mode

The temp file is automatically cleaned up after transmission

Output Window Features:

**Copy All**: Copies all output text to clipboard

**Cancel**: Cancels the in-progress send operation

**Close**: Closes the output window (uses accent color from settings)

Dark Mode: The output window automatically adapts to the application theme (light/dark).

### Fill Key to ECU

The Fill Key to ECU dialog generates MIDS JTRS fill content for Electronic Control Units.

Access: Database page → Expand hierarchical view → Right-click on segment → "Fill Key to ECU"

Dialog Features:

**Tag Types Selection**: Choose MIDS JTRS Tag (IFF is under development)

**Template Selection**: Select from available MIDS JTRS tag templates

**Control/Extension Tag Fields**: View resolved field values and placeholders

**Generated Output**: Shows hexadecimal output after generation

Action Buttons:

| Button | Description |  |
| --- | --- | --- |
| **Generate** | Creates 01B0 FDU fill content from selected template |  |
| **Save** | Saves generated output to a `.bin` file |  |
| **Send to SKL** | Sends the content directly to SKL |  |
| **Cancel** | Closes the dialog |  |

Technical Notes:

Uses 01B0 FDU format only (no EKMS 308 header)

Temporary files use `.8727.bin` extension

Debug output panel shows generation process details

### Database Location

The database is stored at:

%LOCALAPPDATA%\ReceivedKeyProcessorApp\keys.db

Typical locations:

Windows 10/11: `C:\Users\[Username]\AppData\Local\ReceivedKeyProcessorApp\keys.db`

### Backup and Restore

#### Backup

Close the application

Navigate to the database location

Copy the `keys.db` file to a safe location

#### Restore

Close the application

Backup the current `keys.db` file

Copy your backup `keys.db` file to the database location

Restart the application

## Troubleshooting Common Issues

### Application Won't Start

Symptom: Application fails to launch or crashes immediately

Possible Causes:

.NET 8.0 Runtime not installed

Missing required files (EKMS2.exe, DLLs)

Insufficient permissions

Solutions:

Verify .NET 8.0 Windows Desktop Runtime is installed

Check that EKMS2.exe is in the application directory

Run the application as Administrator

Check Windows Event Viewer for error details

### EKMS2.exe Not Found Error

Symptom: Error message "EKMS2.exe not found"

Solutions:

Ensure EKMS2.exe is in the same folder as the application executable

Verify the file name is exactly "EKMS2.exe" (case-sensitive on some systems)

Check file permissions on EKMS2.exe

Reinstall or obtain the correct version of EKMS2.exe

### File Import Fails

Symptom: Unable to import .8727 or .bin files

Possible Causes:

Invalid file format

Corrupted file

EKMS2.exe processing failure

Permission issues

Solutions:

Verify the file is in the correct format (.8727, .8287, .bin, .key)

Check file size (very small or very large files may be invalid)

Ensure EKMS2.exe can access the file

Check that you have write permissions to the temp directory

Review the error log for specific error messages

### Database Access Errors

Symptom: Cannot access or save data to the database

Possible Causes:

Database file locked

Insufficient permissions

Disk full

Database corruption

Solutions:

Close all other instances of the application

Check disk space on the drive where %LOCALAPPDATA% is located

Run the application as Administrator

Restore from backup if database is corrupted

Delete the database file and let the application recreate it (you will lose all data)

### Serial Port Communication Issues

Symptom: Unable to connect to COM port or receive data

Possible Causes:

Wrong COM port selected

Port already in use

Incorrect baud rate

Hardware not connected

Permission issues

Solutions:

Verify the correct COM port number in Device Manager

Close other applications that may be using the port

Check baud rate settings match the equipment

Ensure hardware is properly connected

Run as Administrator for port access

### MIDS JTRS Tag Validation Errors

Symptom: Error when editing or saving MIDS JTRS tags

Possible Causes:

Invalid field values

CRC calculation errors

Spare fields not zero

Internal ID mismatch

Solutions:

Review all field values are within valid ranges

Ensure spare fields are zero (they are not editable)

Verify InternalJTRSKeyID matches in both Control and Extension tags

Check that string fields don't exceed maximum lengths

Cancel edit and try again if errors persist

### ECU KEK ID Mismatch

Symptom: Calculated ECU KEK ID doesn't match expected value

Possible Causes:

Incorrect ShortTitle, Edition, or Segment Number

Different hash algorithm used

Byte ordering differences

Solutions:

Verify ShortTitle, Edition, and Segment Number are correct

Ensure values are left-justified and null-padded

Check that the algorithm matches the specification (SHA-1 with MSB-first byte ordering)

Remember that 0x00000000 is valid for Red Keys

### Poor Tag Detection Score

Symptom: Tag detection shows low confidence score

Possible Causes:

Data doesn't match any known format

Corrupted or incomplete data

Wrong file format

New or custom tag format

Solutions:

Verify you loaded the correct file

Check the file size matches expected format

Review the score breakdown for specific validation failures

Consult format documentation if using custom formats

Consider that the data may be invalid or corrupted

### Performance Issues

Symptom: Application runs slowly or freezes

Possible Causes:

Large files being processed

Database needs optimization

Insufficient memory

Background processes

Solutions:

Close unnecessary applications

Process smaller batches of files

Optimize the database (compact and vacuum)

Increase available RAM

Check for high CPU usage from other processes

### Getting Help

If you continue to experience issues:

**Check Logs**: Review application logs in the application directory

**Review Documentation**: Consult this User Guide and technical documentation

**Contact Support**: Reach out to your system administrator or support team

**Report Bugs**: Provide detailed error messages and steps to reproduce

### Log Files

The application creates log files for troubleshooting:

`RKPA_8727_Import.log` - 8727 file import logs

`RKPA_EKMS2_Service.log` - EKMS2.exe service logs

`ef_core_exceptions.log` - Database operation logs

Logs are typically found in:

Application directory

System temp directory (for secure processing logs)

## Security Considerations

### Data Protection

This application processes sensitive cryptographic data. Follow these security practices:

**Secure Environment**: Run on secure, authorized computers only

**Access Control**: Limit access to authorized personnel

**Backup Security**: Encrypt backups and store securely

**Log Protection**: Protect log files as they may contain sensitive information

**Secure Deletion**: The application securely wipes temporary files

### Best Practices

Never share your database files with unauthorized personnel

Regularly back up your database to secure storage

Use strong passwords for any encryption features

Keep the application and dependencies updated

Report any security concerns immediately

## Appendix: Quick Reference

### Keyboard Shortcuts

| Shortcut | Action |  |
| --- | --- | --- |
| Ctrl+S | Save (when applicable) |  |
| Ctrl+F | Open search |  |
| Ctrl+Z | Undo (in edit mode) |  |
| Escape | Cancel current operation |  |
| F5 | Refresh view |  |

### File Formats Supported

| Extension | Description | Processing |  |
| --- | --- | --- | --- |
| .8727 | Standard 8727 binary | EKMS2.exe → XML → Database |  |
| .8287 | 8287 binary | EKMS2.exe → XML → Database |  |
| .bin | Generic binary | EKMS2.exe → XML → Database |  |
| .key | Key binary | EKMS2.exe → XML → Database |  |
| .xml | XML format | Direct import |  |
| .json | JSON format | Direct import |  |

### Version Information

Current Version: 1.0.106

For the latest version information and updates, contact your system administrator.

End of User Guide